HYPNEXIS'26 · AI-01 · PHASE 2

Kavach Phase 2: real phone calls and partly faked speech

Plan, method and live status. Numbers marked "pending" are filled in automatically from runs/phase2/results.json when each experiment finishes. No number in this document is typed by hand.

# 1. What Phase 2 asks

| # | Requirement | Pass condition |
| --- | --- | --- |
| R1 | Report EER after 8 kHz resampling, two codecs and added noise | Worst case ≤ 2.5 × clean EER |
| R2 | Build 100 partial fakes and locate the synthetic segments | Segment F1 at 0.5 s resolution (report it) |
| R3 | Streaming mode during a call | Score updates every 500 ms |
| R4 | Keep Phase 1 results | Phase 1 EERs within 2 points of before |

# 2. Our approach

## R1 · Channel robustness

Real calls pass through narrowband phone lines and lossy voice codecs, with background noise. We test each condition on its own, on 2,000 clips from 11 In-the-Wild speakers never seen in training:

| Condition | How it is made | Why |
| --- | --- | --- |
| Clean | Silence-trimmed, loudness-normalised 4 s clip | Reference EER |
| 8 kHz resample | Down to 8 kHz and back to 16 kHz | Telephone bandwidth: everything above 4 kHz is lost |
| Codec 1: AMR-NB 12.2 kbps | Real encoder (ffmpeg libopencore_amrnb) at 8 kHz | The standard mobile-network voice codec (GSM/UMTS calls) |
| Codec 2: Opus 12 kbps VoIP | Real encoder (ffmpeg libopus, voip mode) | WhatsApp, Google Meet and most internet calls |
| Added noise | Street/room-like noise at 10 dB SNR | Calls from busy places |
| Random mix | Random chain of the above, plus speaker-to-mic replay | Hardest realistic case |

Training change: the detector is re-trained with random channel augmentation (8 kHz, one of four codecs: AMR, Opus, GSM, MP3, noise at 5–25 dB, speaker-to-mic replay). Because real and fake clips get the same treatment, the channel itself carries no label.

## R2 · Partial fakes and localisation

- Building 100 partial fakes: take a real 6–10 s clip of a test speaker and replace one 1–3 s stretch with a fake clip of the same speaker (In-the-Wild has both for every person). Loudness is matched and the joins get a 20 ms crossfade so there is no click to detect. The exact fake interval is saved as ground truth.

- A segment head: a second small classifier trained on random 1 s crops (clean and channel-augmented) from training speakers only.

- Localisation: slide a 1 s window every 0.5 s. Each 0.5 s cell gets the mean score of the windows covering it. A cell above the threshold is marked synthetic. The threshold is chosen on 60 separate partial fakes from calibration speakers, never on the test set.

- Metric: precision, recall and F1 over 0.5 s cells across all 100 test partial fakes.

## R3 · Streaming mode

- The browser captures microphone audio as raw 16 kHz samples and sends 500 ms chunks over a WebSocket.

- The server keeps a rolling buffer for that call and, every 500 ms, scores the latest 2 s of speech (EER at 2 s was 2.56% in Phase 1).

- The UI shows a live score line, the current verdict and how long the voice has looked synthetic. A smoothed score avoids flicker.

- A "Simulate call" button streams a demo file in real time, so the demo works even without a microphone.

browser mic ──500 ms PCM chunks──► WS /ws/stream

server: rolling 2 s buffer → XLS-R → head → score

◄── {t, score, smoothed, verdict} every 500 ms

## R4 · Phase 1 guard

After every change the Phase 1 experiments are re-run on the same test speakers. The new model is accepted only if each Phase 1 EER is within 2 percentage points of the Phase 1 model.

# 3. Results (filled automatically)

## R1 · Channel robustness

| Condition | EER | × clean |
| --- | --- | --- |
| Clean | 1.80% | 1.00× |
| 8 kHz resample | 1.40% | 0.78× |
| AMR-NB 12.2k | 1.80% | 1.00× |
| Opus 12k | 2.10% | 1.17× |
| Noise 10 dB | 3.50% | 1.94× |
| Random mix | 11.11% | 6.17× |

Worst case: 1.94× clean, target ≤ 2.5× → PASS

## R2 · Localisation

100 partial fakes, 0.5 s cells: precision 79.95%, recall 81.02%, F1 80.48% (threshold 0.560 from calibration fakes).

## R3 · Streaming

Update interval 500 ms; median processing per update 25.7 ms, p95 31.4 ms (must stay below 500 ms).

## R4 · Phase 1 guard

| Phase 1 test | Phase 1 model | Phase 2 model | Change |
| --- | --- | --- | --- |
| Unseen speakers 4 s | 2.31% | 2.01% | -0.30 pts |
| 2 s clips | 2.56% | 2.26% | -0.30 pts |
| 10 s clips | 2.26% | 2.01% | -0.25 pts |
| Phone line | 2.85% | 2.96% | +0.10 pts |

# 4. Build status

| Step | Status |
| --- | --- |
| Codec, 8 kHz, noise and channel-mix functions (real AMR-NB / Opus via ffmpeg) | Done, tested |
| Feature extraction: 2,000 test clips × 6 conditions | Done |
| Feature extraction: channel-augmented training set + 1 s segment crops | Done |
| Re-train detector with channel augmentation + modern TTS + clean real speech | Done |
| 100 + 60 partial fakes, segment head, F1 | Done |
| Streaming WebSocket + live UI + simulate-call | Done |
| Tests for all of the above, deck + document update, final zip | Last |